README.DOC for Crystal Semiconductor Corp. 2.01 Windows 95 Drivers

This Document contains the following:

1. Version Numbers

2. List of files

3. File Summary

4. Installing Crystal Audio Drivers

5. Uninstalling Crystal Audio Drivers

6. Dos Mode

7. Multiple Crystal Audio Cards

1. Version Numbers:

2.01 T2		CWBINIT.EXE

4.03.2010		CWBAUDIO.DRV

4.03.2010		CWBAUDIX.VXD

4.03.2010		CWBFM.DRV

4.03.2010		CWBJOY.VXD

4.03.2010		CWBMIDI.DRV

4.03.2010		CWBMIDI.VXD

4.03.2010		CWBMIX.EXE

4.03.2010		CWBPCPLE.DLL

0.01.000		MSGLOOP.EXE

4.03.2010		SETUP.EXE

0.01.000		VWSTREAM.VXD

0.01.000		WSTREAM.DLL

Microcode:

79   		CS4232 Microcode

6     	CS4232A Microcode

55		CS4236 Microcode

14		CS4236B/CS4237B/CS4238B Microcode

2. List of Files:

CWBPCPLE.DLL        59,392  02-13-97  2:01a

CWBAUDIO.BIN         8,225  02-13-97  2:01a

CWBMIDI.VXD         29,326  02-13-97  2:01a

CWBAUDIO.INF        19,005  02-13-97  2:01a

CWBAUDIO.INI           283  06-17-97  8:55a

CWBAUDIX.VXD        98,840  02-13-97  2:01a

CWBFM.DRV           21,776  02-13-97  2:01a

CWBMIDI.DRV          9,392  02-13-97  2:01a

CWBJOY.VXD          10,157  02-13-97  2:01a

CWBAUDIO.DRV       205,904  02-13-97  2:01a

CWBMIX.EXE         234,858  02-13-97  2:01a

MSGLOOP.EXE          4,112  02-13-97  2:01a

SETUP.EXE          305,664  02-13-97  2:01a

VWSTREAM.VXD         5,175  02-13-97  2:01a

WSTREAM.DLL          9,712  02-13-97  2:01a

CWBINIT.EXE         94,912  02-13-97  2:01a

README.DOC

3. File Summary:

CWBINIT.EXE   	CS4232/36/36b/37b/38b Configuration Program.  A DOS device driver used to initialize and configure the card.  It will download Microcode from CWBAUDIO.BIN.  Using the /X option, it will download resource data from RESOURCE.BIN, CS32RES.BIN, or CS36RES.BIN.

CWBMIX.EXE	DOS mixer used to set or adjust Sound Blaster mixer settings.  It will run as a GUI application or as a command line utility.

CWBAUDIO.BIN  	Data file containing Microcode for the CS4232, CS4232A, CS4236, CS4236B, CS4237B and CS4238B.  It is used by CWBAUDIX.VXD and CWBINIT.EXE.

CWBAUDIO.INI	Default initialization parameters for CWBINIT.EXE.  It is only used if PnP is overridden (/O) or if not using the Win 95 option on a non PnP system (no /W).

CWBAUDIO.INF	Windows 95 installation file.  It will install the files, update the registry, install CWBINIT.EXE /A in the CONFIG.SYS, and it will install CWBINIT.EXE /A in the DOS mode CONFIG.SYS.  It should be modified by OEM's to meet their specific needs.

CWBFM.DRV	Crystal FM MIDI driver.

CWBAUDIX.VXD	CS4232/36/36B/37B/38B Windows 95 virtual device driver.

CWBAUDIO.DRV	CS4232/36/36B/37B/38B Windows 95 wave, aux, and mixer driver.

CWBMIDI.VXD	Crystal MPU-401 MIDI virtual device driver.

CWBMIDI.DRV	Crystal MPU-401 MIDI Driver.

CWBPCPLE.DLL	Windows 95 Power Control Panel extension.  This file will create an extension to the default Windows 95 Power Control Panel, enabling new features such as idle time-out and power down now.  This is not installed by default and requires a change to the install .INF.

CWBJOY.VXD	Crystal Digital Assisted Joystick virtual device driver.

SETUP.EXE	Crystal Windows 95 uninstall program.  Running this program will clean out any Crystal registry entries, delete Crystal files, clean the CONFIG.SYS, and prompt the user to restart.  When Windows 95 restarts, the Crystal drivers should install using the default Windows 95 installation.

VWSTREAM.VXD	It is the main WaveStream engine.  It resides between

WSTREAM.DLL and CWBAUDIX.VXD.  It does the Software 			WaveTable Synthesis, SoundBlaster, and mixing.	This 			VXD is provided in our kit to hook up with the 			Brooktree WaveStream CD.

WSTREAM.DLL		It is the WaveStream component which communicates with 			cwbaudio.drv and in turn passes information down to 			vwstream.vxd.  This DLL is provided in our kit to hook

up with the Brooktree WaveStream CD.

MSGLOOP.EXE		It is a WaveStream component which is provided in our 			kit to hook up with the Brooktree WaveStream CD.

4. Installing Crystal Audio Drivers

1.1.Turn PC off and insert audio card into an ISA slot unless audio device is present in your motherboard.

2.2.Turn PC on and start up Win95.

3.3.Win95 is a PnP OS.  It automatically recognizes new PnP audio hardware and creates a database for the hardware.  The following shows the the Crystal logical devices:

A.Win95 detects the Crystal PnP Audio System Codec.  Win95 asks for the manufacturer's Disk.  Please select the approriate drive.  If the Windows default driver is not grayed out, installing DirectSound games replaces Crystal drivers with Microsoft Windows Sound System drivers.

B.Win95 detects the Disabled Device (Joystick in some platforms).  On certain versions of Windows 95, Windows will prompt for the Win 95 CD in order to re-install the joystick drivers.

C.Win95 detects the Crystal PnP Audio System Control Registers (CS4232 Support Registers in some platforms.)

D.Win95 detects the Crystal PnP Audio System MPU-401 Compatible.

If any of the above (A thru D) is not detected, a previous Win95 sound card installation was done.  For a clean reinstallation, the user should see the Uninstalling Crystal Audio drivers section below.

5. Uninstalling Crystal Audio Drivers

1.From Win95, run setup.exe provided in the Crystal installation diskette. The Setup will do an automatic cleaning up of the Win95 Crystal Audio registry entries. Many problems with reinstallation occur because of existing registry entries from previous installations and because of the \WINDOWS\INF\OEMx.INF files.  After running setup, the various Crystal devices from the Control Panel should disappear.  It is recommended that prior to running Setup, remove the device entries in the Sound, Video, and Game Controller of the System Device Manager, and Crystal entries (if any) in the Other detected devices section of the Device Manager.

2.Shutdown and turn off your PC.

3.Turn PC on, Start Win95, Win95 will now detect new hardware, create an information database, and proceed with the installation of Crystal device drivers.

6. Dos Mode

A.  Restarting the computer in MS-DOS mode.

Shutting down to MS-DOS mode from the Shutdown Win95 Menu will automatically run the CWBINIT.EXE /A in dosstart.bat.  Therefore, shutting down to MS-DOS mode will automatically have the audio card configured for Dos games.

B.  Enabling Dos support in Dos prompt icon.

To enable real DOS mode with CS4236b support:

1.Select a DOS prompt icon.

2.Select Properties.

3.Go to the Program tab.

4.Select the Advanced... button.

5.Click MS-DOS Mode.

6.Click "Specify a new MS-DOS configuration."

7.The following line will be automatically placed into the new MS-DOS configuration (with right path):

DEVICE=C:\WIN95\CWBINIT.EXE /A

7. Multiple Crystal Audio Cards

To enable a CS4236B audio card in a system that

contains multiple Crystal audio cards:

1.  Add the following option to the CWBINIT.EXE

entry in the CONFIG.SYS.

Device=C:\WINDOWS\CWBINIT.EXE /W /Sxx

where xx is the OEM ID number for the Crystal Part

audio card to be enabled.

2.  The OEM ID number can be obtained from the

manufacturer.